Table of Contents

Chapter 7 R2 Signaling. 7-1

7.1 Basic Concepts. 7-1

7.1.1 Line Signaling. 7-1

7.1.2 Register Signaling. 7-7

7.2 Application of R2 Signaling. 7-15

7.3 Basic Signaling Flow. 7-16

# Chapter 7  R2 Signaling

## 7.1  Basic Concepts

Due to the large scale of the telecom network, it is hard to replace channel associated signaling (CAS) with SS7 signaling in a short term. By far, the CAS system is still widely used in the international telecom network and telecom networks of various countries. No. 1 signaling is a subset of R2 signaling.

R2 signaling consists of line signaling and register signaling. Of these two kinds of signaling, the definition varies from country to country.

Line signaling is transmitted between line devices (repeaters). It is composed of line monitoring signals. It is used for monitoring the trunk status and controlling the connection. A line device cannot be shared among trunks. Instead, each trunk must have a line device. Therefore, line signaling is relatively simple to reduce costs, and the types of line signaling are few.

Register signaling is transmitted between registers. It is composed of selection signals and service signals. It is used for selecting route and callee and managing the telephone network. A register is a shared device. Few registers are needed in a signaling network. Therefore, a register can be a complex device for matching more kinds of signaling.

### 7.1.1  Line Signaling

There are three forms of line signaling:

           DC line signaling

           Line signaling with in-band single frequency pulse

           Digital line signaling

#### I. DC Line Signaling

DC line signaling is used for the real line trunks of electromechanical switches. It is actually not used at present. In China, local call networks are all stored program controlled. DC line signaling will not be introduced in this document.

#### II. Line Signaling with In-Band Single Frequency Pulse

In a toll automatic telephone network, if the inter-office transmission system uses carrier, microwave or satellite circuits of frequency-division multiplexing, the audio signal, namely, the in-band single frequency pulse signal, is adopted.

The single frequency used by line signaling is 2600 Hz. It consists of:

           Short signal unit

           Long signal unit

           Continuous signal unit

The short signal unit is a short pulse signal with the nominal value of 150 milliseconds. The long signal unit is a long pulse signal with the nominal value of 600 milliseconds. The nominal interval of sending two signals is 300 milliseconds.

Table 7-1 lists the nominal values of pulse signals and intervals.

Table 7-1 Nominal values of the in-band single frequency pulses and their intervals

| Nominal values of pulse signal or interval |  |  | Sending time deviation at transmitting end (ms) | Recognition time range at receiving end (ms) |
| --- | --- | --- | --- | --- |
| Pulse | Signal length (ms) | Interval (ms) |  |  |
| Short signal unit | 150 | 150 | ± 30 | 80 ± 20 |
| Sending interval | – | 300 | ± 60 | – |
| Long signal unit | 600 | 600 | ± 120 | 375 ± 75 |

There are two kinds of line signaling: forward signaling and backward signaling.

The former is sent from the originating office to the terminating office. And the latter is sent from the terminating office to the originating office. Table 7-2 shows the structure of singling signals.

Table 7-2 Signal structure of line signaling with in-band single frequency pulse

| Serial No. | Connection status (signaling name) |  | Sending direction |  | Signaling signal structure (ms) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Forward | Backward |  |  |
| 1 | Occupation signal |  | → |  | Single pulse 150 |  |
| 2 | Disconnection signal |  | → |  | Single pulse 600 |  |
| 3 | Repeated disconnection signal |  | → |  |  | Used between toll offices and between toll/local offices |
|  |  |  |  |  |  | Used between local offices |
| 4 | Answer signal |  |  | ← | Single pulse 150 |  |
| 5 | Clear signal |  |  | ← | Single pulse 600 |  |
| 6 | Release guard signal |  |  | ← | Single pulse 600 |  |
| 7 | Blocking signal |  |  | ← | Continuous |  |
| 8 | Operator signal | Re-ringing or forced disconnection signal | → |  |  | At least three pulses |
|  |  | Ringback signal |  | ← |  | At least three pulses |
| 9 | Forced release signal |  | A → |  | Single pulse 600 | Equivalent to clear signal |
|  |  |  |  | B ← | Single pulse 600 | Equivalent to release guard signal |

The connection states in Table 7-1 are described as follows:

           Occupation signal is a forward signaling. When the outgoing trunk of the originating office sends an occupation signal, an incoming trunk of the peer office changes its state from idle to occupied.

           Disconnection signal is a forward signaling sent by the outgoing trunk to the incoming trunk of the peer office. It means that the switch can release the call in abnormal call disconnection in addition to normal disconnection. The disconnection signal is sent in any of the following cases:

| 1 | The caller hangs up in call control recovery mode. |
| --- | --- |
| 2 | The operator of original toll office disconnects the call in toll semi-automatic connection. |
| 3 | The original office receives a backward register signaling such as connection busy. |
| 4 | Callee does not pickup after alerting timeout, or caller does not hang up for more than 90 seconds after callee hangs up. |

           The repeated disconnection signal is sent by the outgoing trunk of the original office when it does not receive the release control signal 3 to 5 seconds after it sends the disconnection signal. If the release control signal is still not received after sending the repeated disconnection signal, an alarm is generated.

           The answer signal indicates that the callee picks up the phone. It is a backward signaling sent by the incoming trunk.

           The clear signal indicates that the callee hangs up. It is a backward signaling sent by the incoming trunk from the terminal office to the original office in relay.

           The release control signal is a backward confirmation signal of the disconnection signal. It indicates that the device of the incoming office releases.

           The blocking signal is a backward signaling sent by the incoming trunk of the incoming office. It indicates that the trunk has been blocked.

           The re-ringing signal is a forward operator signaling. After the toll office operator establishes call connection with the callee and the callee answers, if the callee hangs up and the operator needs to call the callee, the operator can send the re-ringing signal.

           The forced disconnection signal is also a forward operator signal. When the toll office operator tries to connect the call, and finds that the callee is engaged in a local call, the operator sends the signal after receiving confirmation from the callee.

           The ringback signal is a backward operator signaling. It is sent by the operator back to the caller.

           The forced release signal is used in the following case. In a bi-directional trunk circuit, sometimes it is occupied in both directions due to disturbances. If no register signaling is received in 15 seconds, one end sends a forward forced release signal (acting as a release signal), and the other end sends a backward forced release signal (acting as a release control signal), and the trunk circuit is released.

#### III. Digital Line Signaling

The line signaling monitors the occupation, release, and blocking state of the trunk lines. To support transmission of 30 voice channel line signaling in the PCM system, a multiframe concept is introduced. A multiframe consists of 16 individual frames, each of which is 125 µs and contains 32 timeslots. A multiframe has 16 TS16. In the TS16 of the frame 0, the first four bits are used for synchronization in the multiframe; the last four bits are used for loss-of-synchronization report. And the TS16 of the other 15 frames are used to transmit the line signaling of the 30 voice channels.

Table 7-3 describes the usage of TS16 in a PCM multiframe.

Table 7-3 Use of TS16 in a PCM multiframe

| Frame 0 | Frame 1 |  | Frame 2 |  | ... |
| --- | --- | --- | --- | --- | --- |
| 00 00 XY XX | abcd Voice channel 1 | abcd Voice channel 16 | abcd Voice channel 2 | abcd Voice channel 17 | … |

           TS16 of frame 0

X: Reserved. It is set to 1.

Y: Loss-of-synchronization report bit. 0 means normal and 1 means loss of synchronization.

           TS16 of other frames

A 30-voice-channel PCM system sends the line signaling by sampling and transmitting the TS16 in a multiframe. There are four bit—a, b, c, and d—available in both transmission directions for each voice channel. Only the first three bits are used for both the forward signaling and the backward signaling.

The bit af, bf, and cf are for the forward signaling, and the ab, bb, and cb are for the backward signaling. Table 7-4 lists the meaning of the signaling.

Table 7-4 Meaning of forward signaling in digital lines

| Bit | Meaning |
| --- | --- |
| af=0 | Caller picks up (occupied) |
| af=1 | Caller hangs up (released) |
| bf=0 | Normal |
| bf=1 | Faulty |
| cf=0 | Operator re-rings or forced releases |
| cf=1 | Operator does not re-ring or forced release |

Table 7-5 Meaning of backward signaling in digital lines

| Bit | Meaning |
| --- | --- |
| ab=0 | Callee picks up |
| ab=1 | Caller hangs up (backward released) |
| bb=0 | Idle |
| bb=1 | Occupied or blocked |
| cb=0 | Operator rings back |
| cb=1 | Operator does not ring back |

Obviously, Cf and Cb are not needed because no operator intervention is needed in:

           Connection between local offices

           Automatic connection between a local office and a toll office

Table 7-6 shows the differences between different digital line signaling in the three bits.

Table 7-6 Signaling bits in the automatic and semi-automatic connection of a toll office

| Connection state | Forward signaling |  |  | Backward signaling |  |
| --- | --- | --- | --- | --- | --- |
|  | af | bf | cf | ab | bb |
| Idle | 1 | 0 | 1 | 1 | 0 |
| Occupied | 0 | 0 | 1 | 1 | 0 |
| Occupation confirmed | 0 | 0 | 1 | 1 | 1 |
| Answer | 0 | 0 | 1 | 0 | 1 |
| Hang up | 0 | 0 | 1 | 1 | 1 |
| Re-ring (forced release) | 0 | 0 | 0 | 1 | 1 |
| Release | 1 | 0 | 1 | 0 | 1 |
| Release control | 1 | 0 | 1 | 1 | 0 |
| Ring back | 1 | 0 | 1 | 0 | 1 |
| Blocked | 1 | 0 | 1 | 1 | 1 |

Refer to the previous paragraphs on the definition of connection states.

### 7.1.2  Register Signaling

#### I. Definition of Register Signaling

The R2 register signaling is in multiple frequency control (MFC) mode. It is divided into two types:

           Forward signaling

           Backward signaling

Both of them are consistent. The forward signaling transmits addresses and controls indications. The backward signaling confirms and controls a call.

When sending a digit, the sending party does not stop sending the forwarding signaling until having received a backward confirmation. Similarly, the receiving end does not stop sending backward signaling until having detected that the peer end stops sending forwarding signaling.

As shown in Figure 7-1, the transmission of R2 register signaling is done in four beats.

Figure 7-1 The transmission process of R2 register signaling

| Beat | Operations |
| --- | --- |
| 1 | The originating office sends the first bit of forwarding signaling. |
| 2 | The terminating office (receiving end) receives and identifies the forwarding signaling, and returns the first bit of backward confirmation signaling. The terminating office thus replies that it has received the forwarding signaling, and informs what forwarding signaling the originating office shall send next. |
| 3 | The originating office receives and identifies the backward confirmation signaling, and stops sending the forward signaling. |
| 4 | The terminating office detects that the peer end stops sending the forward signaling, and stops sending the backward confirmation signaling. When the originating office detects that the peer end stops sending the backward confirmation signaling, it starts the second control period by sending the next bit of forwarding signaling. |

#### II. Coding Mode of MFC Register Signaling

There are 15 types of forward signaling of MFC register signaling. Fifteen combinations of two from the six high frequencies:

           1380 Hz

           1500 Hz

           1620 Hz

           1740 Hz

           1860 Hz

           1980 Hz

There are six backward signaling of MFC register signaling. Six combinations of two from four low frequencies:

           1140 Hz

           1020 Hz

           900 Hz

           780 Hz

Table 7-7 and Table 7-8 details the combination of frequencies.

Table 7-7 Forward signaling

| Code Frq (Hz) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F0 (1380) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F1 (1500) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F2 (1620) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F4 (1740) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F7 (1860) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F11 (1980) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7-8 Backward signaling

| Code Frq (Hz) | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| F0 (1140) |  |  |  |  |  |  |
| F1 (1020) |  |  |  |  |  |  |
| F2 (900) |  |  |  |  |  |  |
| F4 (780) |  |  |  |  |  |  |

#### III. Types and Meanings MFC Register Signaling

As described in the above, the MFC register signaling falls into two types: forward and backward. The forward signaling is further divided into group I and group II. And the backward signaling is further divided into group A and group B. Group A is the acknowledgement of group I. And group B the acknowledgement of group II.

Table 7-9 lists the meanings of the four groups of signaling.

Table 7-9 Meanings of the four groups of signaling

| Forward signal |  |  |  | Backward signal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Group | Name | Meaning | Capacity | Group | Name | Meaning | capacity |
| I | KA | Caller type | 10/15 | A | A Signal | Back control acknowledgement of the number receiving status and connection status | 6 |
|  | KC | Toll connection type | 5 |  |  |  |  |
|  | KE | Toll/local office and urban connection type | 5 |  |  |  |  |
|  | Digital signal | Digit 1–0 | 10 |  |  |  |  |
| II | KD | Originating call service type | 6 | B | B signal | Callee status | 6 |

           Note: For a local office using the step-by-step system, there are 10 user types. For a stored program control (SPC) local office using the crossbar system, there are 15 user types. Forward group I signaling

Forward group I signaling consists of connection control signaling and digital signaling.

For details, refer to Table 7-10 and Table 7-12.

Table 7-10 Forward group I signaling

| Type | Meaning |
| --- | --- |
| KA | It is the caller type signaling sent from the originating local office to the originating toll office or originating international switching center in the forward direction. The purpose of this signaling is to provide such information as:  Charging type (periodical, immediate, free)  User level (ordinary, high priority) The combination of these two kinds of information is expressed with a KA code, as shown in Table 7-11. The high priority user in the table refers to those whose calls take precedence over others’ in the case of network blocking or overload. |
| KC | It is the connection control signaling sent between toll offices in the forward direction. This signaling has the following functions:  Ensures the communication quality of users with high priority.  Completes specified calls.  Connects other specified calls (such as test calls). |
| KE | It is the connection control signaling sent from the terminating toll office to the terminating local office and between local offices in the forward direction. There are two types of KE signaling, as shown in Table 7-11. |
| Digital signaling | It is a selection signaling. The ten digits, 1, 2, 3, …, 0, are used to indicate the calling number, called area code and called number; “15” is used to separate the calling number and called number, indicating the end of the calling number. |

           Backward group A signaling

Backward group A signaling is the MFC signaling of forward group I signaling. It controls and acknowledges forward group I signaling. For details, see Table 7-10 and Table 7-11.

Table 7-11 Backward group A signaling

| Type | Meaning |
| --- | --- |
| A1, A2, A6 | These three kinds of signaling together are called code-sending sequence control signaling. They control the code-sending sequence of forward digital signaling. |
| A3 | A3 is a conversion control signaling for differentiating forward group I from forward group II, and backward group A from backward group B. A3 signaling is the control signaling when it is in:  Toll incoming register at the local office end in the connection from the terminating toll office to the local office  Multi-frequency incoming register of the local call connection In other cases, A3 signaling is a pulse (150±30 ms) signaling. |
| A4, A5 | They are the cause analysis signaling when connection to the callee fails. A4 indicates busy, and A5 an unallocated number. |

Table 7-12 Forward group I signaling and backward group A signaling

|  | Forward group I signaling |  |  |  |  |  |  |  |  |  | Backward group A signaling |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KA code | Contents of KA signaling (including KOA) |  |  |  |  |  | KC code | Contents of KC signaling | KE code | Contents of KE signaling | Digital signaling | Contents of A signaling |  |
|  | Step-by-step local office |  | SPC local office using crossbar system (also including PAM office) |  |  |  |  |  |  |  |  |  |  |
|  | KA |  | KA |  | KA |  |  |  |  |  |  |  |  |
| 1 | Ordinary | Periodical | Ordinary | Periodical | Ordinary | Periodical |  |  |  |  | 1 | A1: send next bit |  |
| 2 |  | User table, immediate |  | User table, immediate |  | User table, immediate |  |  |  |  | 2 | A2: send starting from the first bit |  |
| 3 |  | Printer, immediate |  | Printer, immediate |  | Printer, immediate |  |  |  |  | 3 | A3: shift to B signal |  |
| 4 | Standby |  | Standby |  | Standby |  |  |  |  |  | 4 | A4: telephone key blocking |  |
| 5 | Ordinary, free |  | Ordinary, free |  | Ordinary, free |  |  |  |  |  | 5 | A5: unallocated number |  |
| 6 | Standby |  | Standby |  | Standby |  |  |  |  |  | 6 | A6: send KA and calling number |  |
| 7 | Standby |  | Standby |  | Standby |  |  |  |  |  |  |  |  |
| 8 | Standby |  | High priority, periodical |  | High priority, periodical |  |  |  |  |  |  |  |  |
| 9 | (Have right for suburban automatic call; have right for toll automatic call |  | Standby |  | Standby |  |  |  |  |  |  |  |  |
| 10 | (Have no right for toll/suburban automatic call) |  | High priority, free |  | High priority, free |  |  |  |  |  |  |  |  |
| 11 | – |  | Standby |  |  |  | 11 | Standby | 11* | Voice mailbox notifies the user to leave a message |  |  |  |
| 12 |  |  |  |  |  |  | 12 | “Z” indicates a specified number call | 12 | Standby |  |  |  |
| 13 |  |  | Test call |  |  |  | 13 | “T” indicates a test connection call | 13 | “T” indicates a test call |  |  |  |
| 14 |  |  | Standby |  |  |  | 14 | High priority | 14 | Standby |  |  |  |
| 15 |  |  | – |  |  |  | 15 | Control the number of satellite circuit segments | 15 | Voice mailbox cancels notifying the user to leave a message |  |  |  |
| Note: Those types with brackets are not sent to the originating toll office; “*” indicates that the signal is needed for cooperating with old equipment. |  |  |  |  |  |  |  |  |  |  |  |  |  |

           Forward group II

Forward group II signaling is also called KD signaling. It indicates the originating call service type. It is used, based on KD, to judge whether the attendant can break in or forcefully release a local call. Table 7-14 describes the role of this signaling.

           Backward group B signaling

Backward group B signaling is also called KB signaling. It indicates the status of the callee. It is sent after the receipt of KD signaling to acknowledge the control and connection of KD signaling.

See Table 7-13 for the contents of backward group B signaling.

Table 7-13 Forward group II signaling and backward group B signaling

| Forward group II signaling (KD) |  |  | Backward group B signaling (KB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| KD code | Contents of KD signaling |  | KB code |  | Contents of KB signaling |  |
|  |  |  |  |  | During toll call connection or test call connection (when KD=1, 2 or 6) | During local call connection (when KD=3 or 4) |
| 1 | Semi-automatic call of a toll attendant |  | Used for toll call connection | 1 | Callee idle | Callee idle, first party release recovered |
| 2 | Automatic toll call, |  |  | 2 | Callee local busy | Standby |
| 3 | Urban call |  | Used for urban call connection | 3 | Callee toll busy |  |
| 4 | Fax or user data communication of the urban user; high priority user |  |  | 4 | Telephone key blocked | Callee busy or telephone key blocked |
| 5 | Automatically checking calling number | 5 |  |  | Called number is an unallocated number | Called number is an unallocated number |
| 6 | Test call | 6 |  |  | Standby | Callee idle, calling party release recovered |

Table 7-14 Contents and role of KD signaling

| KD code | Originating call service type | Role of KD signaling |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Whether attendant can break in local call |  | Whether toll attendant can break in |  |
|  |  | Yes | No | Yes | No |
| 1 | Semi-automatic breaking in of toll attendant |  | – | – |  |
| 2 | Automatic toll call | – |  | – |  |
| 3 | Urban call | – |  |  | – |
| 4 | Urban fax or data | – |  | – | – |
| 5 | Automatically checking calling number | – | – | – | – |
| 6 | Test call | – |  | – |  |

## 7.2  Application of R2 Signaling

Figure 7-2 shows the typical application of R2 signaling in NGN.

Figure 7-2 Typical application of R2 signaling in NGN

By packaging R2 messages in H.248 packets, the UMG8900 sends the R2 messages to the SoftX3000 for processing. And thus, NGN is able to interwork with the exchanges and PBXs in PSTN.

## 7.3  Basic Signaling Flow

Figure 7-3 shows the connection process of a local call. The process describes the basic flow of R2 signaling,

Figure 7-3 Signaling process of a local call

In Figure 7-3, the called number is PQRABCD, in which PRQ is the office directional number, and ABCD is the user number. The figure shows that line signaling and register signaling are sent segment by segment. After the transit office receives PQR completely, it starts routing to send register signaling of the originating office. After sending the full number, the originating office waits for the terminating office to send the A3 signal, and then completes the signaling flow. This connection mode takes a long time. Therefore, it is used when the transmission line is of a poor quality.